EQUIPMENT DETAILS

| Equipment Tag: | [MDB-001] | Location: | [Building / Room / Area] |  |
| --- | --- | --- | --- | --- |
| Make / Model: | [e.g. Schneider Prisma / ABB Gemini] | Rated Current: | [___A] |  |
| Supply Voltage: | [___V 3ph+N+E] | No. of Outgoing Circuits: | [Count] |  |
| Last Thermographic Survey: | [DD/MM/YYYY] | Next Survey Due: | [DD/MM/YYYY] |  |
| Protective Device Coordination: | [Verified / Not Verified] | Earth Fault Loop Impedance: | [___Ω (last test) Date:___] |  |

INSPECTION CHECKLIST

| Pass [ ] | Fail [ ] | N/A [ ] | Defect [ ] | Use tick (v) in the frequency column if inspection was performed and result noted in Remarks. |
| --- | --- | --- | --- | --- |

| No. | Inspection / Maintenance Item | Std. | Pre | Daily | Mnthly | Annual | Findings / Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A. VISUAL & PHYSICAL INSPECTION |  |  |  |  |  |  |  |
| A1 | Panel enclosure condition -- no dents, corrosion, or unauthorised modifications | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| A2 | Panel door -- closes and locks securely, gasket intact, IP rating maintained | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| A3 | Cable entry -- all glands sealed, no open knockouts, no vermin ingress | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| A4 | Internal cleanliness -- no dust, moisture, insects, or debris inside panel | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| A5 | Warning labels and circuit schedules -- present, legible, up to date | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| A6 | Single line diagram -- posted inside door, matches actual configuration | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| A7 | Earthing bar connections -- all PE conductors terminated and tight | IEC 60364 | Pre-use | Daily | Monthly | Annual |  |
| B. CIRCUIT BREAKERS & FUSES |  |  |  |  |  |  |  |
| B1 | Main incomer MCCB -- correct rating, not tripped, handle moves freely | IEC 60947 | Pre-use | Daily | Monthly | Annual |  |
| B2 | Outgoing MCBs / MCCBs -- all in correct position (ON/OFF/TRIP) | IEC 60947 | Pre-use | Daily | Monthly | Annual |  |
| B3 | Fuse links (if applicable) -- correct rating, no evidence of blowing | IEC 60269 | Pre-use | Daily | Monthly | Annual |  |
| B4 | Residual Current Device (RCD) -- test button operation monthly | IEC 61008 | Pre-use | Daily | Monthly | Annual |  |
| B5 | Circuit breaker handle exercise -- operate each CB ON-OFF-ON annually | IEC 60947 | Pre-use | Daily | Monthly | Annual |  |
| B6 | Breaker trip settings -- match protection study / load requirements | IEC 60947 | Pre-use | Daily | Monthly | Annual |  |
| C. BUSBARS & TERMINATIONS |  |  |  |  |  |  |  |
| C1 | Busbar connections -- no overheating (check thermally with IR camera) | NETA MTS | Pre-use | Daily | Monthly | Annual |  |
| C2 | Busbar insulation -- no cracking, tracking, or discolouration | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| C3 | Cable terminations -- tight, insulation in good condition, correct ferrules | IEC 60947-7 | Pre-use | Daily | Monthly | Annual |  |
| C4 | Neutral bar connections -- all neutral conductors terminated, no loose wires | IEC 60364 | Pre-use | Daily | Monthly | Annual |  |
| C5 | Thermographic survey -- annual IR scan of all connections and busbars | NETA MTS | Pre-use | Daily | Monthly | Annual |  |
| D. METERING & PROTECTION |  |  |  |  |  |  |  |
| D1 | Voltmeter reading -- within +/-5% of nominal supply | IEC 60038 | Pre-use | Daily | Monthly | Annual |  |
| D2 | Ammeter reading -- within rated current, balanced across phases | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| D3 | Power factor meter (if fitted) -- above 0.9 lagging | IEC 61000 | Pre-use | Daily | Monthly | Annual |  |
| D4 | Energy meter -- recording, check for correct kWh accumulation | IEC 62053 | Pre-use | Daily | Monthly | Annual |  |
| D5 | Surge Protection Device (SPD) -- green indicator, not operated | IEC 61643 | Pre-use | Daily | Monthly | Annual |  |
| D6 | SCADA / BMS integration -- all alarms and measurements transmitting | IEC 61850 | Pre-use | Daily | Monthly | Annual |  |

SIGN-OFF

| Inspected By: | [Name + Position] | Company: | [Company Name] |  |
| --- | --- | --- | --- | --- |
| Signature: |  | Date: | [DD/MM/YYYY] |  |
| Next Inspection Due: | [DD/MM/YYYY] | Work Order Ref.: | [WO No. or N/A] |  |

DISCLAIMER

Generic reference checklist by FreeDocumentsHub.com. Not certified inspection advice. User assumes full responsibility for compliance with applicable local regulations and standards.

FDH-PG-05  |  FreeDocumentsHub.com  |  doers.ifti@gmail.com  |  [DELETE THIS LINE BEFORE USE]