EQUIPMENT DETAILS

| Equipment Tag: | [SDB-001] | Location: | [Building / Floor / Room] |  |
| --- | --- | --- | --- | --- |
| Fed From: | [MDB-001 / Panel Name] | Make / Model: | [e.g. Schneider iC60] |  |
| Rated Current: | [___A] | Supply Voltage: | [___V 3ph+N / 1ph+N] |  |
| No. of Circuits: | [Count] | Last Inspection: | [DD/MM/YYYY] |  |

INSPECTION CHECKLIST

| Pass [ ] | Fail [ ] | N/A [ ] | Defect [ ] | Use tick (v) in the frequency column if inspection was performed and result noted in Remarks. |
| --- | --- | --- | --- | --- |

| No. | Inspection / Maintenance Item | Std. | Pre | Daily | Mnthly | Annual | Findings / Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A. PHYSICAL INSPECTION |  |  |  |  |  |  |  |
| A1 | Panel enclosure -- no damage, IP rating maintained, door seals intact | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| A2 | Panel location -- accessible, 1m clear working space in front | IEC 60364 | Pre-use | Daily | Monthly | Annual |  |
| A3 | Cable entries -- all glands sealed, no bare conductors exposed | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| A4 | Internal cleanliness -- no dust, moisture, or debris | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| A5 | Circuit labels -- all breakers and fuses labelled, schedule posted | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| A6 | Panel earthing -- PE bar connected to main earth, continuity confirmed | IEC 60364 | Pre-use | Daily | Monthly | Annual |  |
| B. PROTECTIVE DEVICES |  |  |  |  |  |  |  |
| B1 | Incomer MCB / MCCB -- correct rating, operational, no thermal damage | IEC 60947 | Pre-use | Daily | Monthly | Annual |  |
| B2 | Outgoing MCBs -- all correctly rated for connected loads | IEC 60947 | Pre-use | Daily | Monthly | Annual |  |
| B3 | RCD / RCCB (if fitted) -- monthly test button operation | IEC 61008 | Pre-use | Daily | Monthly | Annual |  |
| B4 | Arc fault detection (AFDD) -- test function if fitted | IEC 62606 | Pre-use | Daily | Monthly | Annual |  |
| B5 | SPD (surge protector) -- visual check, green status indicator | IEC 61643 | Pre-use | Daily | Monthly | Annual |  |
| C. CONNECTIONS & TERMINATIONS |  |  |  |  |  |  |  |
| C1 | Busbar terminations -- tight, no sign of overheating or oxidation | IEC 60947-7 | Pre-use | Daily | Monthly | Annual |  |
| C2 | Neutral connections -- all neutral wires terminated, not floating | IEC 60364 | Pre-use | Daily | Monthly | Annual |  |
| C3 | Cable condition -- insulation intact, cables supported and not stressed | IEC 60364 | Pre-use | Daily | Monthly | Annual |  |
| C4 | Thermographic inspection -- IR scan annually | NETA MTS | Pre-use | Daily | Monthly | Annual |  |
| C5 | Insulation resistance -- test line-to-earth each circuit (>1 MΩ) | IEC 60364 | Pre-use | Daily | Monthly | Annual |  |
| D. OPERATING MEASUREMENTS |  |  |  |  |  |  |  |
| D1 | Supply voltage -- within +/-5% of nominal | IEC 60038 | Pre-use | Daily | Monthly | Annual |  |
| D2 | Load current -- not exceeding rated incomer current | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| D3 | Phase balance -- current imbalance less than 10% between phases | IEC 61439 | Pre-use | Daily | Monthly | Annual |  |
| D4 | Neutral-to-earth voltage -- less than 1V (no neutral bonding issues) | IEC 60364 | Pre-use | Daily | Monthly | Annual |  |

SIGN-OFF

| Inspected By: | [Name + Position] | Company: | [Company Name] |  |
| --- | --- | --- | --- | --- |
| Signature: |  | Date: | [DD/MM/YYYY] |  |
| Next Inspection Due: | [DD/MM/YYYY] | Work Order Ref.: | [WO No. or N/A] |  |

DISCLAIMER

Generic reference checklist by FreeDocumentsHub.com. Not certified inspection advice. User assumes full responsibility for compliance with applicable local regulations and standards.

FDH-PG-06  |  FreeDocumentsHub.com  |  doers.ifti@gmail.com  |  [DELETE THIS LINE BEFORE USE]