EQUIPMENT DETAILS

| System Tag: | [PV-001] | Location: | [Rooftop / Ground Mount / Carport] |  |
| --- | --- | --- | --- | --- |
| Installed Capacity: | [___kWp] | Inverter Make / Model: | [Brand / Model] |  |
| No. of Panels: | [Count] | Panel Make / Model: | [Brand / Model] |  |
| Commissioning Date: | [DD/MM/YYYY] | Last Cleaning: | [DD/MM/YYYY] |  |
| Last Month Generation (kWh): | [___kWh] | Expected Monthly Yield (kWh): | [___kWh] |  |

INSPECTION CHECKLIST

| Pass [ ] | Fail [ ] | N/A [ ] | Defect [ ] | Use tick (v) in frequency column when inspection performed; note result in Remarks. |
| --- | --- | --- | --- | --- |

| No. | Inspection / Maintenance Item | Std. | Pre | Daily | Mnthly | Annual | Findings / Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- |
| A. SOLAR PANELS |  |  |  |  |  |  |  |
| A1 | Panel surface -- clean, no soiling, dust, bird droppings, or shading | IEC 62446 |  |  |  |  |  |
| A2 | Panel glass -- no cracks, chips, delamination, or discolouration | IEC 62446 |  |  |  |  |  |
| A3 | Panel frames -- no corrosion, mechanical damage, or loose fixings | IEC 62446 |  |  |  |  |  |
| A4 | Panel mounting structure -- all fixings tight, no corrosion on steelwork | IEC 62446 |  |  |  |  |  |
| A5 | Panel orientation and tilt -- unchanged, no obstructions causing shading | IEC 62446 |  |  |  |  |  |
| A6 | Junction boxes on panels -- sealed, no moisture ingress, connectors locked | IEC 62446 |  |  |  |  |  |
| A7 | MC4 connectors -- mated and locked, no UV degradation or damage | IEC 62852 |  |  |  |  |  |
| A8 | EL (electroluminescence) inspection -- hot spots, cracks (annual) | IEC 62446 |  |  |  |  |  |
| B. DC WIRING & COMBINER BOXES |  |  |  |  |  |  |  |
| B1 | DC cables -- insulation intact, UV-rated, no damage or chafing | IEC 62446 |  |  |  |  |  |
| B2 | Combiner box -- fuses/breakers correct rating, connections tight | IEC 62446 |  |  |  |  |  |
| B3 | String open-circuit voltages -- within expected range (Voc per string) | IEC 62446 |  |  |  |  |  |
| B4 | String short-circuit currents -- balanced across all strings (IV trace annual) | IEC 62446 |  |  |  |  |  |
| B5 | DC isolators -- operational, correctly labelled | IEC 62446 |  |  |  |  |  |
| C. INVERTER |  |  |  |  |  |  |  |
| C1 | Inverter output power -- within expected daily yield | IEC 61727 |  |  |  |  |  |
| C2 | Inverter display -- no fault codes, efficiency within spec | IEC 61727 |  |  |  |  |  |
| C3 | Inverter cooling -- vents clear, fans running, no over-temperature alarm | IEC 61727 |  |  |  |  |  |
| C4 | Grid connection -- voltage, frequency, and power factor within limits | IEC 61727 |  |  |  |  |  |
| C5 | Anti-islanding protection -- test annually | IEC 62116 |  |  |  |  |  |
| C6 | Inverter event log -- review for faults, clipping, and curtailment events | IEC 61727 |  |  |  |  |  |
| D. AC SIDE & MONITORING |  |  |  |  |  |  |  |
| D1 | AC output breaker -- correct rating, no trip events | IEC 60947 |  |  |  |  |  |
| D2 | Bidirectional meter -- import and export readings correct | IEC 62053 |  |  |  |  |  |
| D3 | Performance ratio -- calculate monthly (actual yield / expected yield) | IEC 61724 |  |  |  |  |  |
| D4 | SCADA / monitoring -- all parameters transmitting, alerts active | IEC 61724 |  |  |  |  |  |
| D5 | Earthing and bonding -- panels, frames, and inverter all bonded | IEC 62305 |  |  |  |  |  |

DEFECTS & CORRECTIVE ACTIONS

| No. | Item Ref. | Defect Description | Corrective Action | Target Date | Status |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  | [DD/MM/YYYY] | Open / Closed |
| 2 |  |  |  | [DD/MM/YYYY] | Open / Closed |
| 3 |  |  |  | [DD/MM/YYYY] | Open / Closed |
| 4 |  |  |  | [DD/MM/YYYY] | Open / Closed |
| 5 |  |  |  | [DD/MM/YYYY] | Open / Closed |

SIGN-OFF

| Inspected By: | [Name + Position] | Company: | [Company Name] |  |
| --- | --- | --- | --- | --- |
| Signature: |  | Date: | [DD/MM/YYYY] |  |
| Next Inspection Due: | [DD/MM/YYYY] | Work Order Ref.: | [WO No. or N/A] |  |

DISCLAIMER

Generic reference checklist by FreeDocumentsHub.com. Not certified inspection advice. User assumes full responsibility for compliance with applicable local regulations and standards.

FDH-PG-10  |  FreeDocumentsHub.com  |  doers.ifti@gmail.com  |  [DELETE THIS LINE BEFORE USE]