ASME Section IX Quick Reference Chart

Free crash-course reference — FreeDocumentsHub.com   |   Companion to: ASME Section IX Explained — Welding Procedure Qualification from Start to Finish

| WPS · PQR · WPQ — WHO DOES WHAT. Document; What It Proves; Who Relies on It. WPS; Welding instructions for production work; Welders, welding operators, QC reviewers. PQR; Evidence the WPS produces a sound weld; Engineering, QC, third-party auditors. WPQ; Proof a specific welder can execute the WPS; Welding supervisors, QC, AVL/audit reviewers. VARIABLE TYPES. Type; Effect of a Change; Example. Essential; New PQR required before use; Process, P-Number, F-Number change. Suppl. Essential; New PQR only for impact-tested work; Preheat/heat input change, notch-toughness joint. Nonessential; WPS revised; no new PQR; Groove design, fixturing, cleaning method. |  | PROCEDURE QUALIFICATION — 8 STEPS. #; Step. 1; Define code section & process. 2; Draft preliminary WPS (pWPS). 3; Weld the test coupon. 4; NDE the coupon (visual / RT). 5; Run mechanical tests. 6; Record results on the PQR. 7; Finalize and issue the WPS. 8; Qualify the welders (WPQ). MECHANICAL TESTS ON A PQR. Test; Confirms. Tension; Weld/joint strength vs. base metal minimum. Guided bend; Soundness & ductility through the weld. Impact; Notch toughness at minimum design temp. Fillet break / macro; Fusion & soundness, fillet-only quals. |
| --- | --- | --- |

Need your WPS/PQR packages, welder qualification tracking, or full ASME Quality Control Manual prepared? Visit freedocumentshub.com for ASME and Saudi Aramco AVL documentation services.