FREEDOCUMENTSHUB.COM — QUICK REFERENCE CARD

Permit to Work (PTW) Checklist

Separate the person who authorizes a job from the person who performs it. Verify isolation — never assume it. Keep every permit's evidence trail (JSA, gas test, isolation certificate) attached, not filed separately.

Common Permit Types

| Permit | Used For |
| --- | --- |
| Hot Work | Welding, cutting, grinding, sparks/flame |
| Cold Work | General non-spark work |
| Confined Space | Vessels, tanks, pits |
| Excavation | Digging near buried services |
| Electrical / LOTO | De-energizing before work |
| Working at Height | Above fall-risk threshold |
| Lifting | Crane lifts, heavy moves |
| Radiography (NDT) | Gamma/X-ray weld inspection |

Before Work Starts

- JSA / risk assessment completed and reviewed

- Correct permit type selected for the job

- Isolation completed and physically verified

- Gas test completed, within validity window

- PPE specified and confirmed available

- Fire watch assigned (hot work)

- Escape route and emergency response briefed

- Issuer, holder, and area authority signatures obtained

- SIMOPS conflict check completed

During the Work

- Permit displayed and visible at the job site

- Site conditions match the permit

- Periodic gas re-testing where required

- Supervisor / performing authority available on site

- Scope change stops work — new risk assessment required

After Completion

- Work area inspected, left in safe condition

- Isolations removed or re-applied for next job

- Permit signed off and formally closed

- Permit filed with JSA, gas test, isolation certificate

- Lessons learned recorded if anything deviated

⚠ Audit Red Flag Inconsistent timestamps between the permit, gas test certificate, and isolation record is one of the most common nonconformances raised in QA/HSE audits.

Need ASME- and Aramco-AVL-ready documentation? Visit FreeDocumentsHub.com