Welding Symbols Quick Reference Chart

Free crash-course reference — FreeDocumentsHub.com   |   Companion to: Welding Symbols Crash Course — Read Any Welding Drawing in 30 Minutes

| BASIC WELD SYMBOLS. Weld Type; Shape; Typically Used For. Fillet; Right triangle on the line; T-joints, lap joints, corners. Square Groove; Two short parallel lines; Thin-wall butt joints, no prep. V-Groove; Open “V” shape; Beveled butt joints, vessel shells. Bevel-Groove; Half a “V,” one vertical side; One edge beveled. U-Groove; Rounded, cup-shaped channel; Thick-wall butt joints. J-Groove; Half a “U,” one vertical side; Thick-wall, one edge prepped. Plug / Slot; Rectangle; Overlapping plates through a hole. Spot / Projection; Circle; Resistance welds, sheet metal. Seam; Circle with tail lines through it; Continuous resistance weld line. Backing; Rectangle below the line; Backing strip behind a groove weld. Surfacing; Shallow wave-like curved lines; Build-up / cladding on a surface. FINISH METHOD CODES. Code; Method. C; Chipping. G; Grinding. M; Machining. R; Rolling. H; Hammering. P; Peening. U; Unspecified. |  | SYMBOL ANATOMY & DIMENSIONS. Element; Meaning. Reference line; Horizontal line everything attaches to. Arrow; Points from the line to the actual joint. Symbol below line; = ARROW SIDE (weld faces the arrow). Symbol above line; = OTHER SIDE (weld faces away from arrow). Symbol both sides; = weld both faces of the joint. Size (left of symbol); Leg length / groove depth. Length (right of symbol); Run length; blank = full length. Pitch (right, after length); e.g. 50-150 = 50 mm weld every 150 mm. Root opening; Small number inside groove mouth. Included angle; Angle value at groove mouth. Tail (forked end); WPS no., process, spec, NDE reference. SUPPLEMENTARY SYMBOLS. Mark; Meaning. Circle at the bend; Weld ALL AROUND the joint perimeter. Flag at the bend; FIELD WELD — completed on site, not in shop. Flat line on symbol; FLUSH contour. Outward curve on symbol; CONVEX contour. Inward curve on symbol; CONCAVE contour. |
| --- | --- | --- |

Need your WPS/PQR packages, MTR traceability, NDE reports, or full MDR data books prepared? Visit freedocumentshub.com for ASME and Saudi Aramco AVL documentation services.